CAMBRIDGE O LEVEL COMPUTER SCIENCE (2210 / 0478)

TOPIC 2.1: TYPES AND METHODS OF DATA TRANSMISSION

COMPLETE MARK SCHEME & TEACHER ANSWER KEY • 30 MARKS

## Question 1: Data Packets and Packet Structure [6 Marks]

(a) Three parts: 1. Packet Header [1] | 2. Payload [1] | 3. Packet Trailer [1]

(b) Three items in header: Source IP address [1], Destination IP address [1], Packet sequence number [1], (or Packet size [1])

## Question 2: Packet Switching Mechanism [4 Marks]

• File is split into individual numbered packets. [1]

• Packets travel independently via routers along different routes. [1]

• Packets may arrive out of order at the destination. [1]

• Packets are reassembled in the correct order using sequence numbers; checksum is verified. [1]

## Question 3: Direction of Data Transmission [6 Marks]

1. Walkie-Talkies: Half-Duplex [1] — Data travels in both directions, but only one person can speak at a time. [1]

2. Computer to Printer: Simplex [1] — Data travels in one direction only (from computer to printer). [1]

3. Live Video Call: Full-Duplex [1] — Data travels in both directions simultaneously. [1]

## Question 4: Serial vs. Parallel Transmission [6 Marks]

• Bits sent one bit at a time: Serial [1]

• More suitable for long-distance: Serial [1]

• Higher risk of bit skew: Parallel [1]

• Used for internal computer buses: Parallel [1]

(b) Bit skew: When bits sent simultaneously across parallel wires arrive at different times [1], causing data to fall out of sync and become corrupted. [1]

## Question 5: Universal Serial Bus (USB) [4 Marks]

(a) Benefits: Plug and play (automatically detected & drivers loaded) [1], provides power to peripherals [1], backward compatible [1], universal connection standard [1]. (Any 2)

(b) Drawbacks: Cable length limited to ~5m without repeaters [1], slower than internal PCIe buses [1], older standards have lower bandwidth [1]. (Any 2)

## Question 6: Transmission Application Scenario [4 Marks]

(a) Recommendation: Serial transmission [1]

Reason 1: Over long distance (8 km), serial avoids bit skew. [1]

Reason 2: Much cheaper cabling and less electrical interference / crosstalk. [1]

(b) Direction mode: Full-duplex [1]